Risk Management & Compliance Department

Broken Compact Fluorescent Lights (CFL)

Custodial Services/Maintenance Department

Cleanup Steps for Hard Surfaces

 Carefully scoop up glass fragments and powder using stiff paper or cardboard and place debris and paper/cardboard in a glass jar with a metal lid. If a glass jar is not available, use a sealable plastic bag.   (NOTE: Since a plastic bag will not prevent the mercury vapor from escaping, remove the plastic bag(s) from the home after cleanup.)

 Use sticky tape, such as duct tape, to pick up any remaining small glass fragments and powder.  Place the used tape in the glass jar or plastic bag.

 Wipe the area clean with damp paper towels or disposable wet wipes. Place the towels in the glass jar or plastic bag.

 Vacuuming of hard surfaces during cleanup is not recommended unless broken glass remains after all other cleanup steps have been taken. [NOTE: It is possible that vacuuming could spread mercury‐ containing powder or mercury vapor, although available information on this problem is limited.]  If vacuuming is needed to ensure removal of all broken glass, keep the following tips in mind:

* Keep a window or door to the outdoors open;

* Vacuum the area where the bulb was broken using the vacuum hose, if available; and

* Remove the vacuum bag (or empty and wipe the canister) and seal the bag/vacuum debris,

and any materials used to clean the vacuum, in a plastic bag.

 Promptly place all bulb debris and cleanup materials, including vacuum cleaner bags, outdoors in a trash container or protected area until materials can be disposed of.  Avoid leaving any bulb fragments or cleanup materials indoors.

 Next, check with your local government about disposal requirements in your area, because some localities require fluorescent bulbs (broken or unbroken) be taken to a local recycling center. If there is no such requirement in your area, you can dispose of the materials with your household trash.

 Wash your hands with soap and water after disposing of the jars or plastic bags containing bulb debris and cleanup materials.

 Continue to air out the room where the bulb was broken and leave the H&AC system shut off, as practical, for several hours.

Cleanup Steps for Carpeting or Rugs

 Carefully scoop up glass fragments and powder using stiff paper or cardboard and place debris and paper/cardboard in a glass jar with a metal lid. If a glass jar is not available, use a sealable plastic bag. (NOTE: Since a plastic bag will not prevent the mercury vapor from escaping, remove the plastic bag(s) from the home after cleanup.)

 Use sticky tape, such as duct tape, to pick up any remaining small glass fragments and powder.  Place the used tape in the glass jar or plastic bag.

 Vacuuming of carpeting or rugs during cleanup is not recommended unless broken glass remains after all other cleanup steps have been taken.  [NOTE: It is possible that vacuuming could spread mercury‐ containing powder or mercury vapor, although available information on this problem is limited.]  If vacuuming is needed to ensure removal of all broken glass, keep the following tips in mind: o Keep a window or door to the outdoors open;   o Vacuum the area where the bulb was broken using the vacuum hose, if available, and    o Remove the vacuum bag (or empty and wipe the canister) and seal the bag/vacuum debris, and any materials used to clean the vacuum, in a plastic bag.

 Promptly place all bulb debris and cleanup materials, including vacuum cleaner bags, outdoors in a trash container or protected area until materials can be disposed of.   Avoid leaving any bulb fragments or cleanup materials indoors.

 Next, check with your local government about disposal requirements in your area, because some localities require fluorescent bulbs (broken or unbroken) be taken to a local recycling center. If there is no such requirement in your area, you can dispose of the materials with your household trash.

 Wash your hands with soap and water after disposing of the jars or plastic bags containing bulb debris and cleanup materials.

 Continue to air out the room where the bulb was broken and leave the H&AC system shut off, as practical, for several hours.