Round Rock ISD Environmental & Regulatory Compliance Department

Guide to Handling Cathode Ray Tubes (CRTs)/Televisions

This fact sheet provides information on how to properly package and dispose of damaged/broken televisions and monitors that contain cathode ray tubes (commonly known as CRTs).

Do not discard CRT units in the trash! CRT televisions and monitors contain solid lead in the glass and shielding components that make them unsafe and illegal to dispose of in a landfill.

Due to the leaded glass, particular attention must be given to proper packaging for recycling CRT containing devices to limit airborne dust and contact with skin.

Important: Do not attempt to remove any portion of the CRT from its housing or television cabinet.

How to clean up a broken CRT:

- Wear personal protective equipment.

Personal protective equipment helps prevent injury when cleaning up broken CRT glass and debris.

- Recommended attire includes:

safety glasses; puncture-resistant gloves (for example, leather or other heavy duty work gloves); protective clothing such as a long-sleeved shirt; long pants; and closed-toe shoes.

- Use a wet mister to dampen the broken material before you start any cleanup work. Keep the residual glass damp throughout the cleanup process.

- Thoroughly dampen all of the broken CRT material and then carefully scoop up and place the material into a secure container.

- Pick up small glass fragments using powder sticky tape or duct tape.  Do not sweep or vacuum.

- After sticky tape is used on the small glass fragments, wet mop (if working inside building) and clean up the area to prevent residual lead and cadmium particles from becoming airborne.

How to package a broken CRT:

- Use a secure container.

- When packaging broken CRT glass, use a sturdy box to contain all parts, especially the glass. If possible, use double heavy duty plastic bags to hold the CRT broken glass before placing in box.

- Close the box securely and seal it with tape to securely contain all parts. Move the box to a dry, secure location.

- Label the box "Broken TV cathode tube", RRISD Environmental Department, with the date the box was sealed.

- If the CRT/Television glass is broken and cannot be removed immediately, then move the unit to a secure area where it is unlikely to be disturbed.  If stored outside, place in a double heavy duty plastic garbage bag and label as “Broken Glass” Handle with Extreme Care!

- When transporting the broken CRT/Television it should be double bagged using heavy duty bags, and placed in a heavy duty box, sealed and labeled “Broken Glass” Handle with Extreme Care!

- Call Round Rock ISD’s Environmental Department at 512-464-5952 or 512-464-5459 to schedule a pick up for the next available hazardous waste pick up for the broken glass only.

** Keep a “spill kit” on hand:

Use a five-gallon plastic bucket containing two heavy-gauge plastic garbage bags, a Tyvek suit, gloves, dust mask, goggles, dust pan, duct tape, spray bottle and broom. The instruction for cleanup should be taped to the top of the bucket.